FlyPrecise Valve Automation

# Before buying anything

Regular hobby servos (MG995/MG996R) are too weak to turn a 1" brass ball valve. Check how hard ours is to turn: hang a luggage scale on the end of the handle and pull until it moves. Force x distance from the stem = torque. Get a motor with about 2x that.

# Option 1: Buy motorized ball valves (easiest)

- Replace the manual valves with 1" motorized ones (U.S. Solid, 9-24V, about $40 to $70 each).

- Same threads, so they screw right into the current PVC adapters.

- Control with 2 relays from the Arduino. The 5-wire version also tells the Arduino if it's open or closed.

- Downside: slow, about 6 to 8 seconds to open or close.

# Option 2: High-torque servo on our current valves

- Use a 12V 150 kg-cm servo (DS51150) with the motor mount we already have.

- Needs a 3D printed coupler to grab the valve stem and its own 12V power supply (common ground with the Arduino).

- Fast and cheap, but the mount has to be solid or it will flex instead of turning the valve.

# Other ideas

- Solenoid valves: fast and simple, but only on/off, stay powered to hold open, and might not work at our low pump pressure.

- DC gear motor + limit switches: works, but it's basically building Option 1 from scratch.

# Bonus: linking the two valves

- Easiest: in code, always close one valve before opening the other.

- Hardware: replace the tee and both valves with one 3-way motorized valve so only one path can ever be open.

# Videos

- DIY Servo-Controlled Ball Valve

- Simple DIY Servo Valve, 3D Printed

- Water Valve Servo Automation (Voltlog)

- How to wire a 3-wire motorized ball valve

- How to wire a 2-wire motorized ball valve

# Links

- U.S. Solid 1" motorized ball valve (5-wire)

- DS51150 150 kg-cm servo